Lab Practical #01:

Study of basic networking commands and IP configuration.

Practical Assignment #01:

- Perform and explain various networking commands listed below:

  - ipconfig

  - ping

  - getmac

  - systeminfo

  - traceroute / tracert

  - netstat

  - nslookup

  - hostname

  - pathping

  - arp

## ipconfig

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | ipconfig. | Displays basic IP address, subnet mask, and default gateway for all adapters. |
| 2 | ipconfig /all. | Shows detailed configuration for all interfaces including MAC address, DHCP, DNS, etc. |
| 3 | ipconfig /release | Releases the current IP address assigned to your machine. Useful for DHCP. |
| 4 | ipconfig /renew | Renews the IP address from the DHCP server.. |
| 5 | ipconfig /flushdns. | Clears the DNS cache, which can resolve some domain-related issues. |

### Implementation:

## ping

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | ping <host>. | Sends 4 ICMP Echo Requests to the host (default in Windows). Example: ping google.com |
| 2 | ping -t <host>. | Pings the host continuously until you stop it using Ctrl + C.. |
| 3 | ping -n <count> <host>. | Sends a specific number of Echo Requests. Example: ping -n 10 google.com |
| 4 | ping -l <size> <host>. | Sends Echo Request packets with a custom byte size. Example: ping -l 1000 google.com. |
| 5 | ping -4 <host>. | Forces the use of IPv4.. |

### Implementation:

## getmac

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | getmac. | Displays the MAC addresses and connection names of enabled network interfaces. |
| 2 | getmac /v. | Shows a verbose output including MAC address, transport name, and device name.. |
| 3 | getmac /fo list. | Displays output in a list format instead of table. |
| 4 | getmac /fo csv. | Displays output in CSV format, useful for exporting data. |
| 5 | getmac /?. | Shows help and syntax for the command. |

### Implementation:

### 4.systeminfo

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | systeminfo | Displays complete system summary on the console. |
| 2 | systeminfo /s <host>. | Retrieves system info from a remote computer (requires credentials).. |
| 3 | systeminfo /s <host> /u <domain\user> /p <password>. | Connects to a remote system with credentials. |
| 4 | systeminfo /fo table. | Displays output in table format (default).. |
| 5 | systeminfo /fo list. | Displays output in list format. |

### Implementation:

## 5.traceroute/tracert

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | traceroute <hostname or IP> (Linux/macOS) | Traces the route packets take to the destination. |
| 2 | tracert <hostname or IP> (Windows) | Windows equivalent of traceroute.. |
| 3 | tracert -d <hostname>. | Does not resolve hostnames to IP addresses (faster). |
| 4 | tracert -h <max_hops> <hostname> | Sets the maximum number of hops to search for the target. |
| 5 | tracert -w <timeout_in_ms> <hostname> | Sets the timeout (in milliseconds) to wait for each reply. |

### Implementation:

## 6.netstat

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | netstat -a. | Shows all connections and listening ports. |
| 2 | netstat -n. | Displays addresses and port numbers in numeric form (no DNS lookup). |
| 3 | netstat -o. | Shows active connections with the PID (Process ID) using the connection.. |
| 4 | netstat -e. | Displays Ethernet statistics like bytes sent/received.. |
| 5 | netstat -r. | Displays the routing table (similar to route print). |

### Implementation:

## 7.nslookup

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | nslookup <domain> | Looks up the IP address of the given domain (e.g., nslookup google.com). |
| 2 | nslookup <IP address>. | Performs a reverse DNS lookup to find the domain name of an IP. |
| 3 | nslookup <domain> <DNS server> | Uses a specific DNS server for the query. Example: nslookup google.com 8.8.8.8 |
| 4 | set type=MX. | Looks up mail exchange (MX) records for a domain. |
| 5 | set type=NS. | Displays the name servers of a domain. |

### Implementation:

## 8.hostname

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | hostname. | Displays the name of the current computer (host) on the network. |
| 2 | hostname <newname>. | (Linux only) Temporarily sets the new hostname until the next reboot. |
| 3 | hostname -I. | (Linux only) Shows the IP address(es) of the host. |
| 4 | hostnamectl. | (Linux systemd) View or change the permanent hostname and metadata. |
| 5 | hostname /?. | (Windows) Shows help for the command. |

### Implementation:

## 9. pathping

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | pathping <hostname or IP>. | Traces the path and provides packet loss and latency at each hop. |
| 2 | pathping -n <host>. | Stops name resolution, shows only IP addresses (faster). |
| 3 | pathping -h <max_hops>. | Sets the maximum number of hops to search for the destination. |
| 4 | pathping -g <gateway_list> | Uses loose source routing along a specific list of gateways. |
| 5 | pathping -p <ms>. | Sets the wait time between pings (in milliseconds). |

### Implementation:

## arp

### Description:

| No. | Option | Description |
| --- | --- | --- |
| 1 | arp. | Displays help for the arp command.. |
| 2 | arp -a. | Shows the ARP table (IP ↔ MAC address mappings) for all interfaces. |
| 3 | arp -a <IP> | Displays the ARP entry for a specific IP address. |
| 4 | arp -g. | Same as arp -a.. |
| 5 | arp -d <IP>. | Deletes a specific ARP entry from the table.. |

### Implementation: